Variant 1

INCLUDE "lpm_counter.inc";

CONSTANT RED 	= 4;

CONSTANT REDY 	= 6;

CONSTANT GREEN 	= 1;

CONSTANT EMPTY 	= 0;

CONSTANT YELLOW = 2;

SUBDESIGN trafficlight

(

clk			: INPUT;

Leds[3..1] 	: OUTPUT;	-- 3 - RED, 2 - YELLOW, 1 - GREEN

)

VARIABLE

Light : MACHINE WITH STATES (s1,  s2,  s3,  s4,  s5,  s6,

s7,  s8,  s9,  s10, s11, s12,

s13, s14, s15, s16, s17, s18,

s19, s20, s21, s22, s23, s24,

s25, s26, s27, s28, s29, s30,

s31, s32, s33, s34, s35, s36,

s37, s38, s39, s40, s41, s42,

s43, s44, s45, s46, s47, s48,

s49, s50, s51, s52, s53, s54, s55);	-- 55 СТАНИ, БО ТАКИЙ ЦИКЛ 10+2+10+(0,5+0,5)*3+0,5+2

makegertz:lpm_counter WITH (LPM_WIDTH = 24, LPM_MODULUS = 10000000);			-- з cout беремо 2 Гц

BEGIN

makegertz.clock = clk;

Light.clk = clk;--makegertz.cout;

TABLE

Light 	=> Light, 	Leds[];

s1 		=> s2,		RED;

s2 		=> s3,		RED;

s3 		=> s4,		RED;

s4 		=> s5,		RED;

s5 		=> s6,		RED;

s6 		=> s7,		RED;

s7 		=> s8,		RED;

s8 		=> s9,		RED;

s9 		=> s10,		RED;

s10 	=> s11,		RED;

s11 	=> s12,		RED;

s12 	=> s13,		RED;

s13 	=> s14,		RED;

s14 	=> s15,		RED;

s15 	=> s16,		RED;

s16 	=> s17,		RED;

s17 	=> s18,		RED;

s18 	=> s19,		RED;

Variant2

INCLUDE "lpm_counter.inc";

CONSTANT LedRED    = 10;

CONSTANT LedRY     = LedRED+4;

CONSTANT LedGREEN  = LedRY+LedRed;

CONSTANT TIMEMAX   = LedGreen+8+4;

CONSTANT RED 	= 4;

CONSTANT RY 	= 6;

CONSTANT GREEN 	= 1;

CONSTANT NULL 	= 0;

CONSTANT YELLOW = 2;

SUBDESIGN trafficlight3

(

clk			: INPUT;

Led[3..1] 	: OUTPUT;

)

VARIABLE

devider:lpm_counter WITH (LPM_WIDTH = 24, LPM_MODULUS = 10000000);

counter:lpm_counter WITH (LPM_WIDTH = 7, LPM_MODULUS = TIMEMAX);

BEGIN

devider.clock = clk;

counter.clock = devider.cout;

IF counter.q[] <= LedRED THEN Led[] = RED;

ELSIF counter.q[] < LedRY THEN Led[] = RY;

ELSIF counter.q[] <= LedGREEN OR counter.q[] == LedGREEN + 2 OR counter.q[] == LedGREEN + 4 OR counter.q[] == LedGREEN + 6 THEN Led[] = GREEN;

ELSIF counter.q[] >= LedGreen + 8 THEN Led[] = YELLOW;

ELSE Led[] = NULL;

END IF;

END;